Ain Shams University

Faculty of Engineering

Computer and Systems Engineering Department

CSE 311: Computer Organization (2)

MIPS PROCESSOR

Submitted By

Eslam Samir Ali Abo El-Ala

Mohamed Ahmed Anwer Abdelhalim

Nourhan Essam Ahmed Shiba El-Hamd

Shaza Ismail Kaoud

Group 13

Submitted To

Dr. Cherif Salama

Eng. Diaa El-Din Mohamed

Cairo 2015

Table of Contents

List of Figures	v

1.	IMPLEMENTATION DESCRIPTION	1

1.1.	The MIPS processor specs	1

1.2.	Supported Instructions	1

1.3.	Languages used for implementation	1

1.4.	Tools used for implementation and simulation	1

1.5.	Additional tools	1

2.	Datapath	2

3.	Test Programs and their outputs	2

3.1.	A program testing addition and subtraction	2

3.1.1.	Assembly Code	3

3.1.2.	Machine Code in binary	3

3.1.3.	Clock cycles taken	4

3.1.4.	Simulation	4

3.2.	A program testing If condition	5

3.2.1.	C++ Code	5

3.2.2.	Assembly Code	5

3.2.3.	Machine Code in binary	6

3.2.4.	Clock cycles taken	6

3.2.5.	Simulation	6

3.3.	A program testing Multiplication using SLL	7

3.3.1.	C++ Code	8

3.3.2.	Assembly Code	8

3.3.3.	Machine code in binary	8

3.3.4.	Clock cycles taken	8

3.3.5.	Simulation	9

3.4.	A program testing logic operations	9

3.4.1.	C++ Code	9

3.4.2.	Assembly Code	10

3.4.3.	Machine code in binary	11

3.4.4.	Clock cycles taken	11

3.4.5.	Simulation	11

3.5.	A program testing nested loops	13

3.5.1.	C++ Code	13

3.5.2.	Assembly Code	13

3.5.3.	Machine code in binary	14

3.5.4.	Clock cycles taken	14

3.5.5.	Simulation	15

3.6.	A program testing jr instruction	15

3.6.1.	C++ Code	15

3.6.2.	Assembly Code	15

3.6.3.	Machine code in binary	16

3.6.4.	Clock cycles taken	16

3.6.5.	Simulation	16

3.7.	A program initializing an array using loops	17

3.7.1.	C++ Code	17

3.7.2.	Assembly Code	17

3.7.3.	Machine code in binary	17

3.7.4.	Clock cycles taken	18

3.7.5.	Simulation	18

3.8.	A program testing looping and summation	21

3.8.1.	C++ Code	21

3.8.2.	Assembly Code	22

3.8.3.	Machine code in binary	22

3.8.4.	Clock cycles taken	23

3.8.5.	Simulation	23

3.9.	A program testing checking values and branching	25

3.9.1.	C++ Code	25

3.9.2.	Assembly Code	26

3.9.3.	Machine code in binary	26

3.9.4.	Clock cycles taken	26

3.9.5.	Simulation	26

3.10.	A program testing 2D arrays	27

3.10.1.	C++ Code	27

3.10.2.	Assembly Code	27

3.10.3.	Initial data memory in binary	28

3.10.4.	Machine code in binary	28

3.10.5.	Clock cycles taken	28

3.10.6.	Simulation	29

3.11.	A program testing reading initial values from memory & simple operations	33

3.11.1.	Assembly Code	33

3.11.2.	Initial data memory in binary	33

3.11.3.	Machine code in binary	33

3.11.4.	Clock cycles taken	33

3.11.5.	Simulation	34

3.12.	A program testing load and compare	34

3.12.1.	Assembly Code	34

3.12.2.	Initial data memory in binary	35

3.12.3.	Machine code in binary	35

3.12.4.	Clock cycles taken	35

3.12.5.	Simulation	35

4.	Contribution	36

5.	Assembler Manual	37

5.1.	Usage	37

5.2.	Supported instructions	37

5.3.	Supported Data types	37

5.4.	Example of assembly file format	37

6.	REFERENCES	38

List of Figures

Figure 1 MIPS Processor Datapath	2

Figure 2 Program 1 Data Memory	4

Figure 3 Program 1 Register File	5

Figure 4 Program 2 Data Memory	7

Figure 5 Program 2 Register File	7

Figure 6 Program 3 Data Memory	9

Figure 7 Program 3 Register File	9

Figure 8 Program 4 Data Memory	12

Figure 9 Program 4 Register File	13

Figure 10 Program 5 Data Memory	15

Figure 11 Program 5 Register File	15

Figure 12 Program 6 Data Memory	16

Figure 13 Program 6 Register File	17

Figure 14  Program 7 Data Memory	20

Figure 15 Program 7 Register File	21

Figure 16 Program 8 Data Memory	24

Figure 17 Program 8 Register File	25

Figure 18 Program 9 Data Memory	26

Figure 19 Program 9 Register File	27

Figure 20 Program 10 Data Memory	32

Figure 21 Program 10 Register File	32

Figure 22 Program 11 Data Memory	34

Figure 23 Program 11 Register File	34

Figure 24 Program 12 Data Memory	35

Figure 25 Program 12 Register File	36

# IMPLEMENTATION DESCRIPTION

## The MIPS processor specs

- Single cycle MIPS processor alongside an assembler.

- 256 Bytes instruction memory (byte addressable).

- 2 Kbytes data memory (byte addressable).

- 32-bits 32 registers Register File.

- A clock with a clock period 80 ns.

## Supported Instructions

- Arithmetic: add, addi, sub

- Load/Store: lw, sw

- Logic: sll, and, andi, nor

- Control flow: beq, jal, jr

- Comparison: slt

## Languages used for implementation

- Verilog HDL for implementing the MIPS processor.

- Ruby for implementing the assembler. (bonus)

- Assembly and machine code for testing.

## Tools used for implementation and simulation

- Modelsim.

- Active HDL.

- Sublime text.

## Additional tools

- OCRA gem for packaging assembler to exe.

- Git and Github.

# Datapath

Figure 1 MIPS Processor Datapath

# Test Programs and their outputs

## A program testing addition and subtraction

Description: This program adds and subtracts certain values and stores these values in the memory.

### Assembly Code

#initializing data addi $s0, $zero, 4 #$s0 = 4 addi $s1, $s0, 6 #$s1 = 10 addi $s2, $zero, 16 #$s2 = 16 addi $s3, $zero, 68 #$s3 = 68 addi $s4, $zero, 40 #$s4 = 40 #adding and subtracting add $s0, $s3, $s4 #$s0 = (40+68) = 108 add $s4, $s1, $s3 #$s4 = (68+10) = 78 addi $s1, $s2, -10 #$s1 = (16-10) = 6 slt $s3, $s4, $s1 #$s3 = 0 slt $s2, $s3, $s0 #$s2 = 1 #storing data into memory sw $s0, 4($s3) #$s0 in loc 4 in memory (0+4) .. loc 4 contains 108 sw $s1, 8($s3) #$s1 in loc 8 in memory (0+8) .. loc 8 contains 6 sw $s3, 4($s0) #$s3 in loc 112 in memory (108+4) .. loc 112 contains 0 lw $s5, 8($s3) #$s5 = 6 addi $s5, $s5, 2 #$s5 = 8 sw $s2, 12($s5) #$s2 in loc 20 in memory (12+8) .. loc 20 contains 1 sw $s4, 20($s5) #$s4 in loc 28 in memory (20+8) .. loc 28 contains 78

*we expect that location 4 in memory contains 108 in decimal or 6C in hexadecimal, location 8 contains 6, location 20 contains 1, location 28 contains 78, and location 112 contains 0.

### Machine Code in binary

00100000 00010000 00000000 00000100 00100010 00010001 00000000 00000110 00100000 00010010 00000000 00010000 00100000 00010011 00000000 01000100 00100000 00010100 00000000 00101000 00000010 01110100 10000000 00100000 00000010 00110011 10100000 00100000 00100010 01010001 11111111 11110110 00000010 10010001 10011000 00101010 00000010 01110000 10010000 00101010 10101110 01110000 00000000 00000100 10101110 01110001 00000000 00001000 10101110 00010011 00000000 00000100 10001110 01110101 00000000 00001000 00100010 10110101 00000000 00000010 10101110 10110010 00000000 00001100 10101110 10110100 00000000 00010100

### Clock cycles taken

17 Clock Cycles

### Simulation

Figure 2 Program 1 Data Memory

*The memory is byte addressable, so location 4 contains 0x6C (128 in decimal), location 8 contains 0x06, location 20 contains 0x01, location 28 contains 0x4E (78 in decimal), and location 112 contains 0x00. those values are similar to the values expected before.

Figure 3 Program 1 Register File

## A program testing If condition

Description: This program tests if condition and branch.

### C++ Code

i=0; j=14; if (i<j) { A[i] = 1000; } i=20; if (i<j) { A[i] = 2000; }

### Assembly Code

#initializing data addi $s0, $zero, 4 #$s0 = 4 addi $s1, $zero, 0 #$s1 = 0 addi $s2, $s0, 10 #$s2 = 14 sw $s1, 0($s0) #$s1 stored in location 4 .. 0 in loc 4 sw $s2, 4($s0) #$s2 stored in location 8 .. 14 in loc 8 #if condition lw $t0, 0($s0) #$t0 = 0 lw $t1, 4($s0) #$t1 = 14 slt $t2, $t0, $t1 #$t2 = 1 beq $t2, $zero, exit #wont branch sll $t0, $t0, 2 #$t0*4 = 0 add $t0, $t0, $s0 #$t0 = 4 addi $s5, $zero, 1000 #$s5 = 1000 sw $s5, 0($t0) #put 1000 in loc 4 addi $t0, $t0, 20 #$t0 = 24 slt $t2, $t0, $t1 #$t2 = 0 beq $t2, $zero, exit #Branch sll $t0, $t0, 2 #$t0*4 = 96 .. wont be executed add $t0, $t0, $s0 #$t0 = 100 .. wont be executed sw $s5, 0($t0) #put 1000 in loc 100 .. wont be executed exit:

*we expect location 4 in memory to have 1000 in decimal (0x3E8), location 8 to have 14 (0xE), and location 100 to have 0.

### Machine Code in binary

00100000 00010000 00000000 00000100 00100000 00010001 00000000 00000000 00100010 00010010 00000000 00001010 10101110 00010001 00000000 00000000 10101110 00010010 00000000 00000100 10001110 00001000 00000000 00000000 10001110 00001001 00000000 00000100 00000001 00001001 01010000 00101010 00010000 00001010 00000000 00001010 00000001 00000000 01000000 10000000 00000001 00010000 01000000 00100000 00100000 00010101 00000011 11101000 10101101 00010101 00000000 00000000 00100001 00001000 00000000 00010100 00000001 00001001 01010000 00101010 00010000 00001010 00000000 00000011 00000001 00000000 01000000 10000000 00000001 00010000 01000000 00100000 10101101 00010101 00000000 00000000

### Clock cycles taken

16 Clock Cycles

### Simulation

Figure 4 Program 2 Data Memory

*The memory is byte addressable, so location 4 contains 0x3E8 (1000 in decimal), location 8 contains 0xE, location 100 contains 0x0. those values are similar to the values expected before.

Figure 5 Program 2 Register File

## A program testing Multiplication using SLL

Description: This program loops 10 times and multiplies A by 2 each loop, initially A equals 2.

### C++ Code

A = 2; for (i=0 ; i<10 ; i++) { A = A*2; }

### Assembly Code

#initializing data addi $s0, $zero, 2 #$s0 = 2 -> A addi $s1, $zero, 0 #$s1 = 0 -> i addi $s2, $zero, 10 #$s2 = 10 addi $s3, $zero, 0 #$s3 = 0 # Multiply and Divide loop: sll $s0, $s0, 1 #$s0 = $s0*2 addi $s1, $s1, 1 beq $s1, $s2, end jal loop end: sw $s0, 0($s3) #A is stored in location 0 .. loc 0 contains 2048 (0x800)

*Initially A was equal to 2, the loop loops 10 times and each time it multiplies A by 2, hence, the final expected result is 2*(2^10) = 2048 (0x800) which is stored in location 0 in memory.

### Machine code in binary

00100000 00010000 00000000 00000010 00100000 00010001 00000000 00000000 00100000 00010010 00000000 00001010 00100000 00010011 00000000 00000000 00000000 00010000 10000000 01000000 00100010 00110001 00000000 00000001 00010010 01010001 00000000 00000001 00001100 00000000 00000000 00000100 10101110 01110000 00000000 00000000

### Clock cycles taken

44 clock cycles.

### Simulation

Figure 6 Program 3 Data Memory

*the data memory contains 0x800 in location 4 as expected before (memory is byte addressable).

Figure 7 Program 3 Register File

## A program testing logic operations

Description: this program does some logic operations on data stored in registers.

### C++ Code

A = 215; B = 255; C = 157; D = 0; E = A&B; F = A&C; G = A&D; H = A!¦B; I = A!¦C; J = A!¦D; H = A&5;

### Assembly Code

#initializing data addi $s0, $zero, 215 #$s0 = 215 -> A addi $s1, $zero, 255 #$s1 = 255 -> B addi $s2, $zero, 157 #$s2 = 157 -> C addi $s3, $zero, 0 #$s3 = 0 -> D #Logic operations and $t0, $s0, $s1 #$t0 = 215 = 00000000000000000000000011010111 (0x000000D7) and $t1, $s0, $s2 #$t1 = 149 = 00000000000000000000000010010101 (0x00000095) and $t2, $s0, $s3 #$t2 = 0 = 00000000000000000000000000000000 (0x00000000) nor $t3, $s0, $s1 #$t3 = 32 = 11111111111111111111111100100000 (0xFFFFFF00) nor $t4, $s0, $s2 #$t4 = 40 = 11111111111111111111111100101000 (0xFFFFFF20) nor $t5, $s0, $s3 #$t5 = 40 = 11111111111111111111111100101000 (0xFFFFFF28) andi $t6, $s0, 5 #$t6 = 05 = 00000000000000000000000000000101 (0x00000005) #Storing data sw $t0, 0($s3) sw $t1, 4($s3) sw $t2, 8($s3) sw $t3, 16($s3) sw $t4, 20($s3) sw $t5, 24($s3) sw $t6, 28($s3)

*As we can see from the commented code, the expected value in location 0 in memory is 0x000000D7, in location 4 is 0x00000095, in location 8 is 0x00000000, in location 16 is 0xFFFFFF00, in location 20 is 0xFFFFFF20, in location 24 is 0xFFFFFF28, and in location 28 is 0x00000005.

### Machine code in binary

00100000 00010000 00000000 11010111 00100000 00010001 00000000 11111111 00100000 00010010 00000000 10011101 00100000 00010011 00000000 00000000 00000010 00010001 01000000 00100100 00000010 00010010 01001000 00100100 00000010 00010011 01010000 00100100 00000010 00010001 01011000 00100111 00000010 00010010 01100000 00100111 00000010 00010011 01101000 00100111 00110010 00001110 00000000 00000101 10101110 01101000 00000000 00000000 10101110 01101001 00000000 00000100 10101110 01101010 00000000 00001000 10101110 01101011 00000000 00010000 10101110 01101100 00000000 00010100 10101110 01101101 00000000 00011000 10101110 01101110 00000000 00011100

### Clock cycles taken

18 clock cycles

### Simulation

Figure 8 Program 4 Data Memory

*the data memory contains values 0x800 in location 0 in memory is 0x000000D7, in location 4 is 0x00000095, in location 8 is 0x00000000, in location 16 is 0xFFFFFF00, in location 20 is 0xFFFFFF20, in location 24 is 0xFFFFFF28, and in location 28 is 0x00000005, as expected before(memory is byte addressable).

Figure 9 Program 4 Register File

## A program testing nested loops

Description: This program has to loops, each loops from 0 till 5, the program counts number of equal elements in both loops.

### C++ Code

int n=0; for (int i=0;i<5;i++) { for(int j=0;j<5;j++) { if(i==j) { n++; } } }

### Assembly Code

#initializing data addi $s0, $zero, 0 #n=0 addi $s1, $zero, 0 #i=0 addi $s2, $zero, 0 #j=0 addi $s3, $zero, 5 addi $s4, $zero, 0 L1: addi $s2, $zero, 0 #j=0 L2: beq $s1, $s2, equal jal skip equal: addi $s0, $s0, 1 skip: addi $s2, $s2, 1 beq $s2, $s3, L3 jal L2 L3: addi $s1, $s1, 1 beq $s1, $s3, exit jal L1 exit: sw $s0, 0($s4) #loc 0 contains 5

*We expect location 0 in memory to have value 5 (0x05), which is the number of times I was equal to j.

### Machine code in binary

00100000 00010000 00000000 00000000 00100000 00010001 00000000 00000000 00100000 00010010 00000000 00000000 00100000 00010011 00000000 00000101 00100000 00010100 00000000 00000000 00100000 00010010 00000000 00000000 00010010 01010001 00000000 00000001 00001100 00000000 00000000 00001010 00100010 00010000 00000000 00000001 00100010 01010010 00000000 00000001 00010010 01110010 00000000 00000001 00001100 00000000 00000000 00000111 00100010 00110001 00000000 00000001 00010010 01110001 00000000 00000001 00001100 00000000 00000000 00000101 10101110 10010000 00000000 00000000

### Clock cycles taken

145

### Simulation

Figure 10 Program 5 Data Memory

*the data memory contains value 0x05 in location 0 in memory, as expected before(memory is byte addressable).

Figure 11 Program 5 Register File

## A program testing jr instruction

Description: This program loops, and uses jr instruction in jumping.

### C++ Code

for(int i=0;i<5;i++) { j++; }

### Assembly Code

addi $s0, $zero, 0 #j=0 addi $s1, $zero, 0 #i=0 addi $s2, $zero, 5 addi $s3, $zero, 16 addi $s4, $zero, 0 addi $s0, $s0, 1 #j++ addi $s1, $s1, 1 #i++ beq $s1, $s2, exit jr $s3 #jump to instruction number 4 exit: sw $s0,0($s4)

*The data memory is expected to contain 0x05 (as the loop loops 5 times so j = 5) in location 0.

### Machine code in binary

00100000 00010000 00000000 00000000 00100000 00010001 00000000 00000000 00100000 00010010 00000000 00000101 00100000 00010011 00000000 00010000 00100000 00010100 00000000 00000000 00100010 00010000 00000000 00000001 00100010 00110001 00000000 00000001 00010010 01010001 00000000 00000001 00000010 01100000 00000000 00001000 10101110 10010000 00000000 00000000

### Clock cycles taken

29 clock cycles.

### Simulation

Figure 12 Program 6 Data Memory

*the data memory contains value 0x05 in location 0 in memory, as expected before(memory is byte addressable).

Figure 13 Program 6 Register File

## A program initializing an array using loops

Description: this program initializes an array from the second element with value = first element + 100.

### C++ Code

for (int i=1;i<100;i++) { A[i] = h+B[0]; }

### Assembly Code

addi $s0, $zero, 12 #base address of A =12 addi $s5, $zero, 5 #A[0] = 5 addi $s2, $zero, 100 #h = 100 (0x64) addi $s3, $zero, 1 #i=1 addi $s4, $zero, 100 addi $s1, $zero, 0 sw $s2, 0($s1) #h = 100 in loc 0 sw $s5, 0($s0) #A[0] = 5 in loc 12 loop: lw $t0, 0($s0) #$t0 = A[0] = 5 add $t0, $t0, $s2 #$t0 = A[0] +h = 105 sll $t1, $s3, 2 #$t1 = i*4 add $t1, $t1, $s0 #$t1 = i*4 + base of A sw $t0, 0($t1) #store 105 in loc $t1 addi $s3, $s3, 1 #i++ beq $s3, $s4, exit #if(i==100) {goto exit;} jal loop #jump to loop exit:

*The expected values in memory are: location 0 contains 100 (0x64), location 12 contains 5 (0x05) (first element in the array), and the array will be stored starting from the second element from location 16 till location 408 each containing (0x69).

### Machine code in binary

00100000 00010000 00000000 00001100 00100000 00010101 00000000 00000101 00100000 00010010 00000000 01100100 00100000 00010011 00000000 00000001 00100000 00010100 00000000 01100100 00100000 00010001 00000000 00000000 10101110 00110010 00000000 00000000 10101110 00010101 00000000 00000000 10001110 00001000 00000000 00000000 00000001 00010010 01000000 00100000 00000000 00010011 01001000 10000000 00000001 00110000 01001000 00100000 10101101 00101000 00000000 00000000 00100010 01110011 00000000 00000001 00010010 10010011 00000000 00000001 00001100 00000000 00000000 00001000

### Clock cycles taken

799 clock cycles.

### Simulation

Figure 14  Program 7 Data Memory

*the data memory contains values: location 0 contains 100 (0x64), location 12 contains 5 (0x05) (first element in the array), and the array will be stored starting from the second element from location 16 till location 408 each containing (0x69), as expected before(memory is byte addressable).

Figure 15 Program 7 Register File

## A program testing looping and summation

Description: this program initializes an array with ones then adds the elements of the array to each other.

### C++ Code

i=0; sum = 0; for(int i=0;i<10;i++) { sum += A[i]; }

### Assembly Code

#initializing array addi $s0, $zero, 12 #base address of A =12 addi $s5, $zero, 1 #A[0] = 1 addi $s3, $zero, 1 #i=1 addi $s4, $zero, 10 sw $s5, 0($s0) #A[0] = 1 in loc 12 loop: lw $t0, 0($s0) #$t0 = A[0] = 1 sll $t1, $s3, 2 #$t1 = i*4 add $t1, $t1, $s0 #$t1 = i*4 + base of A sw $t0, 0($t1) #store 1 in loc $t1 addi $s3, $s3, 1 #i++ beq $s3, $s4, endini #if(i==10) {goto endini;} jal loop #jump to loop #summation endini: addi $s0, $zero, 0 #i=0 addi $s1, $zero, 0 #sum=0 addi $s2, $zero, 12 #base address of A =12 addi $s3, $zero, 9 label: sll $t0, $s0, 2 #$t0 = i*4 add $t0, $t0, $s2 #$t0 = i*4 + base lw $t0, 0($t0) add $s1, $s1, $t0 #sum = sum + A[i] addi $s0, $s0, 1 #i++ slt $t0, $s3, $s0 beq $t0, $zero, label

*We expect to find 1 in location 12, 1 in each location after that from location 16 till 48 (10 elements of the array each of value 1), the sum is not stored in memory so we expect to find it in register $s1, where register $s1 is expected to hold 10 in decimal (0xA).

### Machine code in binary

00100000 00010000 00000000 00001100 00100000 00010101 00000000 00000001 00100000 00010011 00000000 00000001 00100000 00010100 00000000 00001010 10101110 00010101 00000000 00000000 10001110 00001000 00000000 00000000 00000000 00010011 01001000 10000000 00000001 00110000 01001000 00100000 10101101 00101000 00000000 00000000 00100010 01110011 00000000 00000001 00010010 10010011 00000000 00000001 00001100 00000000 00000000 00000101 00100000 00010000 00000000 00000000 00100000 00010001 00000000 00000000 00100000 00010010 00000000 00001100 00100000 00010011 00000000 00001001 00000000 00010000 01000000 10000000 00000001 00010010 01000000 00100000 10001101 00001000 00000000 00000000 00000010 00101000 10001000 00100000 00100010 00010000 00000000 00000001 00000010 01110000 01000000 00101010 00010000 00001000 11111111 11111000

### Clock cycles taken

150

### Simulation

Figure 16 Program 8 Data Memory

*The data memory is as expected before, we can see that it contains 1from location 12 till location 48.

Figure 17 Program 8 Register File

*The register file is as expected before, we can see that register $s1 (register number 17) contains 0xA which is 10 in decimal (summation of the 10 elements of the array).

## A program testing checking values and branching

Description: this program checks a value using if condition and sets another value based on this check.

### C++ Code

A=5; B=5; if(A<=B) { A=10; }

### Assembly Code

addi $s1, $zero, 5 #A=5 addi $s2, $zero, 5 #B=5 addi $s3, $zero, 0 slt $s0, $s2, $s1 #if(A<=B) #{$s0 = 0;} beq $s0, $zero, label jal exit label: addi $s1, $zero, 10 #A=10 sw $s1, 0($s3) #loc 0 contains 10 exit:

*Location 0 is expected to have 10 (0x0A).

### Machine code in binary

00100000 00010001 00000000 00000101 00100000 00010010 00000000 00000101 00100000 00010011 00000000 00000000 00000010 01010001 10000000 00101010 00010000 00010000 00000000 00000001 00001100 00000000 00000000 00001001 00100000 00010001 00000000 00001010 10101110 01110001 00000000 00000000

### Clock cycles taken

7 clock cycles.

### Simulation

Figure 18 Program 9 Data Memory

*Location 0 contains 0x0A as expected, (Memory is byte addressable).

Figure 19 Program 9 Register File

## A program testing 2D arrays

### C++ Code

sum = 0; for(int i = 0; i < 4; i++) { for(int j = 0; j < 5; j++) { A[i][j] = B[j][i]; sum = sum + A[i][j]; } }

### Assembly Code

#Test Program #10 - 2D arrays .data .word 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20 .text addi $s0, $zero, 0 # sum = 0 addi $t0, $zero, 0 # i = 0 addi $t1, $zero, 0 # j = 0 addi $s1, $zero, 0 # for B-array starting from first memory location addi $s2, $zero, 100 # to start saving array A from byte no. 100 addi $s3, $zero, 3 addi $s4, $zero, 4 L1: addi $t1, $zero, 0 L2: lw $t2, 0($s1) sw $t2, 0($s2) addi $s1, $s1, 4 addi $s2, $s2, 4 add $s0, $s0, $t2 addi $t1, $t1, 1 slt $t3, $s4, $t1 beq $t3, $zero, L2 addi $t0, $t0, 1 slt $t4, $s3, $t0 beq $t4, $zero, L1

Program is expected to copy values from first 20 word to words starting from 100 and keep the sum in a register.

### Initial data memory in binary

00000000 00000000 00000000 00000001 00000000 00000000 00000000 00000010 00000000 00000000 00000000 00000011 00000000 00000000 00000000 00000100 00000000 00000000 00000000 00000101 00000000 00000000 00000000 00000110 00000000 00000000 00000000 00000111 00000000 00000000 00000000 00001000 00000000 00000000 00000000 00001001 00000000 00000000 00000000 00001010 00000000 00000000 00000000 00001011 00000000 00000000 00000000 00001100 00000000 00000000 00000000 00001101 00000000 00000000 00000000 00001110 00000000 00000000 00000000 00001111 00000000 00000000 00000000 00010000 00000000 00000000 00000000 00010001 00000000 00000000 00000000 00010010 00000000 00000000 00000000 00010011 00000000 00000000 00000000 00010100

### Machine code in binary

00100000 00010000 00000000 00000000 00100000 00001000 00000000 00000000 00100000 00001001 00000000 00000000 00100000 00010001 00000000 00000000 00100000 00010010 00000000 01100100 00100000 00010011 00000000 00000011 00100000 00010100 00000000 00000100 00100000 00001001 00000000 00000000 10001110 00101010 00000000 00000000 10101110 01001010 00000000 00000000 00100010 00110001 00000000 00000100 00100010 01010010 00000000 00000100 00000010 00001010 10000000 00100000 00100001 00101001 00000000 00000001 00000010 10001001 01011000 00101010 00010000 00001011 11111111 11111000 00100001 00001000 00000000 00000001 00000010 01101000 01100000 00101010 00010000 00001100 11111111 11110100

### Clock cycles taken

183 clock cycles.

### Simulation

Figure 20 Program 10 Data Memory

Figure 21 Program 10 Register File

register[16] is keeping the sum int(210) as expected and values in memory.

## A program testing reading initial values from memory & simple operations

### Assembly Code

#Test Program #11 - LW & operations .data .word 15, 16 .text addi $t0, $zero, 0 addi $t1, $zero, 4 lw $s0, 0($t0) lw $s1, 0($t1) add $s3, $s0, $s1 nor $t3, $s0, $s1 sll $s4, $s0, 2

Program is expected to load 2 values from memory and in registers keep their sum, nor, and the first shifted left by 2 -multiplied by 4-.

### Initial data memory in binary

00000000 00000000 00000000 00001111 00000000 00000000 00000000 00010000

### Machine code in binary

00100000 00001000 00000000 00000000 00100000 00001001 00000000 00000100 10001101 00010000 00000000 00000000 10001101 00110001 00000000 00000000 00000010 00010001 10011000 00100000 00000010 00010001 01011000 00100111 00000000 00010000 10100000 10000000

### Clock cycles taken

7 clock cycles.

### Simulation

Figure 22 Program 11 Data Memory

Figure 23 Program 11 Register File

Results as expected.

## A program testing load and compare

### Assembly Code

#Test Program #12 - LW & compare #pseudo code: Is b < a .data .word 5, 15 .text addi $t0, $zero, 0 addi $t1, $zero, 4 lw $s0, 0($t0) lw $s1, 0($t1) slt $s2, $s1, $s0

Program is expected to load 2 values and compare them setting register $s2 by 0 if second is greater than first value, and by 1 if second is less than first.

### Initial data memory in binary

00000000 00000000 00000000 00000101 00000000 00000000 00000000 00001111

### Machine code in binary

00100000 00001000 00000000 00000000 00100000 00001001 00000000 00000100 10001101 00010000 00000000 00000000 10001101 00110001 00000000 00000000 00000010 00110000 10010000 00101010

### Clock cycles taken

5 clock cycles.

### Simulation

Figure 24 Program 12 Data Memory

Figure 25 Program 12 Register File

Registers values as expected after running the program.

# Contribution

Eslam Samir Ali Abo El-Ala [ID: 1200259]

- ALU Module.

- Adder, Subtractor, SltCircuit, And, Nor, and Shiftleft Modules.

- Mux and sign extender Modules.

- ALU Control Unit Module.

Mohamed Ahmed Anwer Abdelhalim [ID: 1101803]

- Instruction Memory Module

- Data Memory Module

- Assembler [Bonus]

Nourhan Essam Ahmed Shiba El-Hamd [ID: 1201605]

- Control unit Module

- Program counter Module

- 9 Test programs [Bonus]

Shaza Ismail Kaoud [ID: 1200717]

- Register file Module

- Clock Module

- Delays formatting

- 3 Test programs [Bonus]

MIPS module, report & design was collaborative effort in team meetings.

# Assembler Manual

the assembler is a simple command line tool written in Ruby and packaged to exe using “OCRA” gem.

## Usage

“assembler.exe ASSEMBLY_FILE” OR “ruby assembler.rb ASSEMBLY_FILE”

This will generate binary code file(s) in the same folder of the ASSEMBLY_FILE

You can view the usage from the command line by calling:
“assembler.exe” OR “ruby assembler.rb”

## Supported instructions

- Arithmetic: add, addi, sub

- Load/Store: lw, sw

- Logic: sll, and, andi, nor

- Control flow: beq, jal, jr

- Comparison: slt

## Supported Data types

.word

## Example of assembly file format

.text # this is an ignored comment and $s0, $s1, $s2 label: addi $s1, $s2, -2 # another ignored comment jal label jr $ra # another allowed comment sll $t0, $t1, 4 exit: .data .word 12, 14 .word -15

NOTE: labels are not supported in data segment, it will raise a syntax error

# REFERENCES

[1] “Computer Organization and Design”, 5th edition, David A. Patterson, John L. Hennessy.

[2] Lectures Slides, Dr. Cherif Salama, 2015.